Numerical Admissibility and Regularization of Shear-Suppression Closures for Reduced Tokamak Transport

Leon Sandler

Independent researcher, Northbrook, IL 60062 USA (e-mail: sandler.leon@gmail.com; ORCID: 0009-0007-4584-808X)

Abstract—Closures in which turbulent heat transport is suppressed by sheared plasma flow make the transport coefficient depend on the evolving profile, yet the conditions under which they define a solvable problem are rarely examined. This paper studies such closures in a reduced model of a reactor-scale tokamak, with a one-dimensional radial energy balance, fusion heating, and a toroidal-rotation equation, so that only the closure changes between runs. The local closure, whose shearing rate contains the second derivative of the temperature, is found numerically to be ill posed: its steady state depends on how the edge is treated, and a grid-scale instability appears at a threshold that grows as the square root of the number of cells. A closure that smooths the shearing rate over a fixed length is stable and grid converged over the tested conditions; its fusion gain converges at second order and is fitted by an inverse-square relation in the suppression threshold. For momentum transport, a steady flux relation gives closed-form conditions under which a shear-dependent viscosity admits a steady rotation profile and predicts its saturation, fold, and hysteresis, which direct solutions reproduce to ten digits. The baseline, with its edge temperature imposed, is consistent with an L-mode scaling (a check, not a validation), the smoothed closure raises the fusion gain by 0.6% at a threshold of 0.3 and by 6% at 0.1, and neutral-beam torque of reactor size adds about 1.5%. Parameter uncertainty is quantified. No experimental data are used.

Index Terms—Bifurcation, fusion reactors, numerical stability, plasma confinement, plasma transport processes, tokamaks.

I. INTRODUCTION

Predicting energy confinement in a tokamak requires a model of the transport driven by micro-turbulence. The transport is stiff: above a critical normalized temperature gradient a small increase of the gradient produces a large increase of the heat flux, so profiles are pinned near marginal stability [1], [2]. Sheared E×B flow decorrelates the turbulence and reduces the transport; this underlies the edge transport barrier and many improved-confinement regimes [3]–[6], and quench rules that suppress the turbulence when the shearing rate exceeds a growth-rate scale are used in transport models [7]. A closure that encodes this is state dependent: the shearing rate is a function of the evolving profile, and the feedback it creates is the standard route to bistability and hysteresis in models of the L–H transition [8], [9].

Whether such a closure defines a well-behaved steady problem, and how much it changes the fusion gain under controlled conditions, are separate questions that are seldom answered together. This paper makes four contributions.

1)  Closed-form admissibility conditions for shear-dependent flux closures, from a steady flux relation, with the saturation, fold, and hysteresis predicted when they fail (Section III), tested against direct solutions (Section V-C).

2)  A diagnosis of the local heat closure: because the shearing rate contains the second derivative of the temperature, the steady problem is numerically ill posed, and a smoothed (adaptive-field) closure with a fixed length restores numerical stability and convergence over the tested conditions (Sections III-B and V-A).

3)  A controlled, verified comparison in which only the closure differs, with grid-converged results, a fitted gain relation, a torque-driven extension, and a parameter-uncertainty study (Sections V-B to V-E).

4)  A physical calibration: the baseline is benchmarked against recognized confinement scalings, operating limits, and neutral-beam torque, and the conclusions are stated as conditional on the unknown suppression threshold (Section VI).

Nothing is fitted to experiment, no new fluid equation is proposed, and the model describes no specific device; the results are statements about closures in a reduced model. “Well posed” is used here in a numerical sense: the admissibility conditions of Section III are proved, but for the nonlinear heat problem existence, uniqueness, and continuous dependence are not, and the evidence is numerical (eigenvalues, convergence orders, edge-condition variants).

II. MODEL

A. Energy Balance and Baseline Closure

One temperature T(r) (electrons and ions equal) is solved on a circular torus with flux-surface radius r ∈ [0, a], major radius R0, and fixed density n(r) and safety factor q(r). The thermal energy density is 3nT, so a diffusivity χ acting on the energy per particle gives the heat flux −3nχ ∂rT (T in energy units):

3n∂tT = 1r∂r3r n χ∂rT + Saux + Sα − Srad.	(1)

The sources are a Gaussian auxiliary heating of power Paux, alpha heating Sα = (n²/4)⟨σv⟩Eα for a 50:50 deuterium–tritium plasma with the Bosch–Hale reactivity [10], and bremsstrahlung Srad = 5.35×10−37 n²T1/2 W m−3 (T in keV). The fusion power is Pfus = 5Pα, the gain Q = Pfus/Paux, and τE = W/(Paux + Pα − Prad). The edge temperature is fixed at Ta = 4 keV. The model reduces the single-fluid magnetohydrodynamic description [11], [12] to a thermal-transport coefficient; no fluid equation is modified. Table I lists the parameters, which are generic reactor-scale values, not a model of a specific device. The baseline diffusivity is a smoothed critical-gradient form [1], [2],

χbase = χn + χsw ln1 + expκ − κcw,   κ = R0|∂rT|T.	(2)

B. State-Dependent Closure and the Adaptive Field

The closure divides the baseline by a shear-suppression factor,

χ = χbase1 + Asc2,   A − ℓ21r∂rr∂rA = ωEγ02,	(3)

where γ0 = cs/R0 is a decorrelation-rate proxy (cs = (T/mi)1/2, mi = 2.5 amu), ωE is the Hahm–Burrell shearing rate [4], ωE = |(r/q) ∂r(qEr/rB)| with Er = (en)−1∂r(nT), and A is a dimensionless adaptive field that relaxes to (ωE/γ0)² and diffuses over the length ℓ (zero-flux ends); it is the steady limit of ∂tA = DA∇²A + [(ωE/γ0)² − A]/τA with ℓ² = DAτA. The threshold sc is the only other parameter, and sc → ∞ recovers the baseline. The limit ℓ → 0 is the local closure, A = (ωE/γ0)². The reference length ℓ = 0.05 m is about 16 ion gyroradii at 10 keV; it is a new parameter, varied in Section V-E.

C. Rotation, Viscosity Closures, and Coupling

Toroidal rotation Ω(r, t) obeys

ρmR02∂tΩ = 1r∂rrμeffR02∂rΩ + τin,   μeff = μ0FΛ,   Λ = rqγ0|∂rΩ|,	(4)

with ρm = nmi, regularity on the axis, and Ω(a) = 0. The torque density τin has a Gaussian profile of width 0.4a and total ∫τindV = Ttot; μ0 = Pr ρ̄ χs with Pr = 1 unless stated (μ0 = 3.11×10−7 kg m−1 s−1). Λ = ωrot/γ0 is the rotation shearing rate in units of γ0. Four viscosity closures F(Λ) are used: F = 1; 1/(1 + Λm); the floor form f + (1 − f)/(1 + Λ²); and f + (1 − f)exp(−αΛ²). Rotation adds to the diamagnetic shear in the heat closure because qEr/rB contains −Ω: ωE → |ωdia + σωrot| with σ = +1 when the shears add and σ = −1 when they oppose, and the coupling is one way (T sets γ0; Ω adds shear).

TABLE I

Model parameters held identical in every run

| Quantity | Value |
| --- | --- |
| R0, a, B | 6.2 m, 2.0 m, 5.3 T |
| n(r), q(r) | 1020(1 − 0.75(r/a)²) m−3; 1 + 2(r/a)² |
| Ta; Paux | 4 keV (fixed); 40 MW Gaussian, width 0.4a |
| χn, χs, κc, w | 0.3, 1.0 m² s−1, 4, 0.5 |
| mi; Eα | 2.5 amu; 3.5 MeV |
| ℓ (reference) | 0.05 m |
| μ0 (Pr = 1) | 3.11×10−7 kg m−1 s−1 |
| Torque profile | Gaussian, width 0.4a, 0–250 N m |

III. ADMISSIBILITY AND REGULARIZATION

A. Shear-Dependent Viscosity: Steady Flux Relation

Proposition 1 (steady flux relation). In steady state, with regularity on the axis, the shearing rate satisfies at each radius

FΛΛ = Θr,   Θ = Irqγ0μ0R02,   Ir = ∫0τ(r′) r′ dr′.	(5)

The closure enters only through Ψ(Λ) = ΛF(Λ), the normalized momentum flux; Θ is the torque number, proportional to the torque (Θmax = 2.017×10−3 per N m here).

Proposition 2 (admissibility). (a) Ψ′ = F(1 + d ln F/d ln Λ), so Ψ increases if and only if d ln F/d ln Λ > −1. (b) If Ψ is increasing with supremum Ψ∞, the relation has exactly one solution at each radius if and only if Θ < Ψ∞: a steady state exists for every torque when Ψ∞ = ∞ and only for Θmax < Ψ∞ otherwise (saturation). (c) If Ψ has a local maximum ΨM then a local minimum Ψm < ΨM, three solutions exist for Ψm < Θ < ΨM; the branch started at Λ = 0 ends in a fold at Θ = ΨM, and on decreasing torque the upper branch persists to Ψm: a hysteresis window [Ψm, ΨM] in Θ.

Proposition 3 (families). F = 1/(1 + Λm) is admissible iff m ≤ 1 (m = 1 saturates at Ψ∞ = 1; for m > 1 the fold is at Λ* = (m − 1)−1/m, Ψ = Λ*/m, i.e. Θ = 1/2 for m = 2). F = f + (1 − f)/(1 + Λ²) is admissible iff f > 1/9. F = f + (1 − f)exp(−αΛ²) is admissible iff f > 2e−3/2/(1 + 2e−3/2) = 0.3086, for any α. Proofs are in the supplement. This is the condition under which a nonlinear diffusion problem with a drive-dependent flux is well posed [13], and part (c) is the rotation analogue of the S-curve of L–H models [8], [9].

Proof sketch. Integrating the steady rotation equation from the axis, where rG vanishes by regularity, gives rG = −I(r), hence μ0R0²F(Λ)|∂rΩ| = I/r and, with Λ = r|∂rΩ|/qγ0, Eq. (5). Proposition 2 follows from Ψ′ = F + ΛF′ and the monotonicity of Ψ. For F = 1/(1 + Λm), d ln F/d ln Λ = −mΛm/(1 + Λm) ∈ (−m, 0], which stays above −1 iff m ≤ 1; for the floor forms Ψ′ = f + (1 − f)g with min g = −1/8 (algebraic) or −2e−3/2 (exponential), giving f > 1/9 and f > 0.3086. Full proofs are in the supplement.

TABLE II

Admissibility of the viscosity closures

| Closure F(Λ) | Steady state for all Θ? | Threshold |
| --- | --- | --- |
| 1; 1/(1 + Λm), m < 1 | yes | – |
| 1/(1 + Λ) | no: saturates | Θ < 1 |
| 1/(1 + Λ²) | no: fold | Θ < 1/2 |
| f + (1 − f)/(1 + Λ²) | yes if f > 1/9 | 1/9 = 0.1111 |
| f + (1 − f)exp(−αΛ²) | yes if f > 0.3086 | 0.3086, any α |

B. Heat Closure: Branching and the Need for Regularization

The heat flux of the local closure is Φ(T′, T″) = 3nχbaseT′/(1 + ωE²/sc²γ0²) with ωE = ω0(r, T, T′) + cT″, c = 10³/B V per keV (T in keV). Because Φ depends on ωE only through ωE², the integrated balance rΦ(T′, T″) = −Ih(r) (with Ih the net source inside r) determines T″ only up to the choice of branch ωE = ±ω*: the steady local problem is a branching second-order equation, and a solution may switch branch in a thin layer. Nothing in the Dirichlet edge value or in regularity on the axis selects the branch, so the choice is made by the discretization of the edge cell. This is not a loss of ellipticity: the principal part, Deff = [∂Φ/∂T′ + ∂r(∂Φ/∂T″)]/3n, stays at least 0.30 m² s−1 at every radius for all sc tested (Table S4 of the supplement). The smoothed closure of Eq. (3) removes the branching: Φ is a smooth functional of the profile through A, and (T, A) obey a closed system of two second-order equations with the conditions T(a) = Ta, T′(0) = 0, and A′(0) = A′(a) = 0. Sections V-A and V-B test this diagnosis; it is supported by the evidence there but not proved.

IV. NUMERICAL METHOD AND VERIFICATION

The energy equation is discretized conservatively on N uniform cells (finite volume, face conductivities from the arithmetic mean of the cell values). All edge and axis derivatives are second order: the cell gradient at the edge cell and the edge-face derivative use the quadratic through the edge value and the last two cells, the axis cell uses T = A + Br², and Er/r is extrapolated evenly to the axis. Steady states are obtained by damped Newton iteration with a banded (local) or dense (smoothed) central-difference Jacobian, started from a backward-Euler march of the baseline and followed in sc by continuation; linear stability is read from the eigenvalues of the Jacobian of dT/dt. The rotation equation is solved by scaled Newton iteration with torque continuation, and torque ramps by backward-Euler marching so that the dynamics select the branch. Details are in the supplement.

TABLE III

Verification (all checks are reproduced by the code archive)

| Check | Result |
| --- | --- |
| Bosch–Hale at 10, 20 keV | 1.136, 4.33 ×10−22 m³ s−1 |
| Power balance residual | max 2×10−10 of heating (sc = ∞ to 0.03) |
| sc = 109 vs baseline | difference 0 keV |
| Radau integration vs Newton | max 1×10−10 keV (4 values of sc) |
| Order of convergence of Q | 1.7–2.0 (baseline and sc ≥ 0.05) |
| λ1(sc = 2), N = 100, 400 | -2.675, -2.675 s−1 |
| Flux relation vs direct solution | max relative error 5×10−11 (17 cases) |

V. RESULTS

A. The Local Closure Is Numerically Ill Posed

With the second-order edge treatment and no smoothing, the steady state exists but its linear stability depends on the resolution: at every sc below a threshold sclin(N) a grid-scale mode localized in the last cells has a positive growth rate (Table IV, Fig. 1a,b). The growth rate rises by roughly an order of magnitude per doubling of N, and the threshold grows as N0.52, so for any finite sc an unstable resolution is reached. In time integration at N = 100 a cold start of the local closure settles for sc ≥ 0.3 and runs away below it (Fig. 1).

TABLE IV

Local closure: onset of the grid-scale instability against resolution

| N | sclin | λ1(sc = 0.4) (s−1) | λ1(sc = 0.2) (s−1) |
| --- | --- | --- | --- |
| 50 | 0.124 | -2.67 | -2.64 |
| 100 | 0.181 | -2.67 | -2.63 |
| 200 | 0.261 | -2.67 | 1.49e+05 |
| 400 | 0.372 | -2.67 | 2.45e+06 |

The converged value depends on how the edge is treated (Table V, Fig. 1c). A first-order half-cell edge treatment, which damps the grid-scale mode, appears stable and even produces a fold at sc = 0.047; this disappears when the edge is treated consistently, so it is an artifact. Imposing an explicit extra edge condition (the last cell takes its neighbour's shearing rate, or no suppression in the last cell) makes the local closure stable and convergent, but the two answers differ by 10% at sc = 0.05. The smoothed closure agrees with the edge-held condition to 0.3% at sc = 0.1 and varies by only 0.8% over a tenfold range of ℓ.

TABLE V

Gain Q at sc = 0.1 for different edge treatments and smoothing lengths (baseline Q = 3.702)

| Closure (edge condition) | Q, N = 200 | Q, N = 400 | λ1, N = 400 |
| --- | --- | --- | --- |
| local, first-order edge | 4.142 | 4.134 | -2.18 |
| local, second-order edge | 4.051 | 4.056 | 7.48e+06 |
| local, edge value held | 3.924 | 3.930 | -2.53 |
| local, none in last cell | 3.856 | 3.862 | -2.6 |
| smoothed, ℓ = 0.02 m | 3.935 | 3.932 | -2.53 |
| smoothed, ℓ = 0.05 m | 3.920 | 3.919 | -2.54 |
| smoothed, ℓ = 0.10 m | 3.908 | 3.907 | -2.55 |
| smoothed, ℓ = 0.20 m | 3.899 | 3.899 | -2.57 |

Fig. 1. Local closure. (a) Leading eigenvalue λ1 against sc for four resolutions (signed logarithm). (b) Threshold sclin against N. (c) Q at sc = 0.1 for different edge conditions (orange: local; blue: smoothed).

B. Smoothed Closure: Convergence and Controlled Comparison

The smoothed closure is linearly stable at every sc tested (λ1 ≤ -0.91 s−1, down to sc = 0.02) and converges at second order: the observed order is 1.73–2.05 over sc = 0.5 to 0.05 (Fig. 3b), and Q changes by 0.04% between N = 400 and N = 800 at sc = 0.05. A cold start settles on the steady state for every sc tested. Table VI gives the comparison. The gain is described by a fitted inverse-square relation, ΔQ = C/sc² with C = 0.00206 (range 0.00204–0.00209 for sc ≥ 0.2), i.e. ΔQ/Q = 0.056%/sc²; the fit is within 12% over 0.02 ≤ sc ≤ 1 for this parameter set. It is an empirical relation for the tested range, not a derived or universal scaling law. The confinement time barely changes (τE = 1.816 s at sc = 0.05 against 1.836 s), so the gain acts through the power balance and the core temperature (T0 rises from 15.2 to 16.9 keV). Like any discretization, the smoothed closure is resolution limited at very strong coupling: a grid-scale mode turns unstable below sc = 0.034, 0.025, and 0.015 at N = 100, 200, and 400, and is absent down to sc = 0.015 at N = 800. In contrast with the local closure, this threshold falls as the grid is refined, so it is a resolution requirement (ℓ must be resolved), not a property of the model.

TABLE VI

Smoothed closure (ℓ = 0.05 m, N = 400) at 40 MW: gain, core temperature, and benchmark quantities

| sc | Q | ΔQ/Q | T0 (keV) | H98 | βN |
| --- | --- | --- | --- | --- | --- |
| ∞ | 3.702 | – | 15.2 | 0.48 | 1.05 |
| 1 | 3.704 | 0.06% | 15.3 | 0.48 | 1.05 |
| 0.5 | 3.710 | 0.22% | 15.3 | 0.48 | 1.05 |
| 0.3 | 3.725 | 0.62% | 15.3 | 0.48 | 1.05 |
| 0.2 | 3.754 | 1.4% | 15.3 | 0.48 | 1.05 |
| 0.1 | 3.919 | 5.9% | 15.7 | 0.48 | 1.07 |
| 0.07 | 4.153 | 12.2% | 16.1 | 0.49 | 1.10 |
| 0.05 | 4.577 | 23.6% | 16.9 | 0.50 | 1.14 |
| 0.03 | 5.909 | 59.6% | 19.7 | 0.54 | 1.28 |
| 0.02 | 8.222 | 122.1% | 24.7 | 0.59 | 1.49 |

Fig. 2. Smoothed closure at 40 MW (N = 400, ℓ = 0.05 m): (a) temperature, (b) diffusivity, and (c) smoothed shearing rate against r/a for the baseline and sc = 0.3 and 0.1. The suppression is confined to the edge.

Fig. 3. Smoothed closure. (a) Relative gain against sc with the fitted inverse-square relation (dashed) and the 5–95 % interval from the parameter-uncertainty study (bars). (b) Convergence of Q with N (dotted: N−1; dashed: N−2). (c) Rank correlation of the gain at sc = 0.1 with each sampled parameter.

C. Tests of the Admissibility Theory

The rotation equation was solved directly (finite volume, N = 200, baseline temperature profile, torques 25–250 N m) and compared with Proposition 1 (Fig. 4b): the simulated Λmax agrees with the solution of F(Λ)Λ = Θ(r) to a relative error of at most 4.8×10−11 over 17 cases, and a steady state exists exactly when Θmax < Ψ∞. The last torque with a steady state is 247.8 N m for m = 2 (Θmax = 0.49995; predicted fold at 1/2) and 495.7 N m for m = 1 (Θmax = 0.99999; predicted saturation at 1). For the floor closure with f = 0.05 < 1/9 the theory gives ΨM = 0.5280 and Ψm = 0.4235, a hysteresis window of 210–262 N m; torque ramps show the jump between 260 and 280 N m and a return at or below 210 N m, while f = 0.20 shows no hysteresis (Fig. 4c).

Fig. 4. Admissibility theory. (a) Normalized flux Ψ = ΛF(Λ); the fold for m = 2 is marked. (b) Λmax from direct solutions against the flux relation. (c) Torque ramps for the floor closure (f = 0.05: hysteresis; shaded: predicted window; f = 0.20: none).

D. Torque-Driven Shear

TABLE VII

Gain with torque-driven shear (sc = 0.5, N = 100, 40 MW, shears adding unless noted)

| Torque (N m) | Θmax | Q const. | Q, m = 1 | Q, m = 2, f = 0.2 | Q const., σ = −1 |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 3.71 | 3.71 | 3.71 | 3.71 |
| 50 | 0.100 | 3.81 | 3.83 | 3.81 | 3.76 |
| 100 | 0.199 | 4.06 | 4.22 | 4.08 | 3.97 |
| 200 | 0.384 | 4.95 | 6.44 | 5.26 | 4.80 |
| 250 | 0.470 | 5.57 | 8.67 | 6.39 | 5.41 |

With the shears adding, the gain rises monotonically with torque (Table VII, Fig. 5): for a constant viscosity Q goes from 3.71 to 4.06 at 100 N m and 4.95 at 200 N m (34% above the baseline); a viscosity that falls with shear amplifies the effect (m = 1: Q = 6.44 at 200 N m). The inadmissible closure m = 2 has no steady rotation profile above Θ = 1/2 (250 N m), as Proposition 3 predicts, whereas the admissible floor closure does. When the shears oppose, the gain is smaller but still increases. At the neutral-beam torque of Section VI-C (about 36 N m) the gain is 1.5% at sc = 0.5 and 9.3% at sc = 0.2.

Fig. 5. Gain against torque for three viscosity closures (solid: shears adding; dotted: opposing). The shaded band is the full-energy neutral-beam torque estimate for a reactor-scale device (Section VI-C).

E. Parameter Uncertainty and Sensitivity

A scrambled Sobol sequence (fixed seed, 128 points) samples χs ∈ [0.7, 1.4] m² s−1, κc ∈ [3.6, 4.4], w ∈ [0.3, 0.7], Ta ∈ [3, 5] keV, n0 ∈ [0.8, 1.2]×1020 m−3, B ∈ [4.8, 5.8] T, and ℓ ∈ [0.02, 0.10] m (128 converged). The baseline gain Q varies from 1.6 to 7.1 (5–95 %), but the relative gain is far more stable: 0.5–0.8% at sc = 0.3, 4–8% at 0.1, and 18–32% at 0.05 (Fig. 3a). The rank correlations (Fig. 3c) identify the parameters that control the gain at sc = 0.1. A second sequence over the torque study (torque 25–200 N m, profile width 0.2–0.6a, Pr 1–4, sc 0.2–1, both signs, ℓ) gives a gain of 0–37% (median 4%) over 64 converged samples.

VI. PHYSICAL CALIBRATION AND ENGINEERING APPLICABILITY

The parameters of the model are illustrative; this section places them against recognized scalings and limits, using an ITER-like reference device (R0 = 6.2 m, a = 2.0 m, B = 5.3 T, plasma current 15 MA, elongation 1.7, 2.5 amu). The plasma current is not an input of the one-dimensional model, so it is taken from the reference device; all formulas are given in the code archive.

A. Confinement Scalings

With the net heating Pnet = 66.2 MW and the line-average density 7.5×1019 m−3, the baseline confinement time 1.84 s is 1.01 times the ITER89-P L-mode scaling [14] (1.81 s) and 0.48 times the IPB98(y,2) H-mode scaling [15] (3.85 s). The stiff baseline with a fixed 4 keV edge is therefore an L-mode-like plasma. The agreement with ITER89-P is a consistency check, not an independent validation: the edge temperature is imposed, the critical-gradient parameters are generic, and the density and heating profiles are prescribed, so other parameter choices would also bring H89 near unity. The smoothed closure acts only on the edge shear, and even at sc = 0.02 it raises the H98 factor from 0.48 to 0.59 (Table VI): it produces a partial edge barrier, not an H-mode pedestal, and must not be read as predicting H-mode performance.

B. Operating Limits

The Greenwald fraction n̄/nG = 0.63 [16], with nG = Ip/πa² = 1.19×1020 m−3, is fixed by the density profile. The normalized beta βN = 1.05 for the baseline rises to 1.07 at sc = 0.1 and 1.49 at sc = 0.02, below the Troyon-type limit of order 3 [17]; the magnetohydrodynamic limit therefore does not bound the closure in the range studied, although the model contains no stability physics, so this is a consistency check only.

C. Neutral-Beam Torque

The heating neutral beams of a reactor-scale device deliver 33 MW at 1 MeV (deuterium) [18]. A beam of power P and speed v injects a torque T = 2PRt/v, since each particle of energy mv²/2 carries angular momentum mvRt; with v = 9.79×106 m s−1 and a tangency radius Rt = 4.5–6 m this gives 30–40 N m (full-energy component only, no losses), consistent with the statement that the torque scales as P/E1/2 and is small in a large device [18]. The torque scan to 250 N m is therefore a sensitivity range up to 7 times the estimate, not a prediction; at the estimated torque the gain is 1.5% (sc = 0.5) to 9.3% (sc = 0.2), and the Mach number at 250 N m is 0.75.

D. The Suppression Threshold

The threshold plays the role of the maximum linear growth rate in units of γ0 in a quench rule [7], sc = γmax/γ0. The largest smoothed diamagnetic shear in the model is ωE/γ0 ≈ 0.08, so for sc ≳ 0.3, i.e. growth rates above 0.3cs/R0, the closure changes Q by less than 1%, and it changes it by more than 10% only for sc ≲ 0.07. Which regime applies cannot be decided without gyrokinetic growth rates for the profiles in question, and that calibration, together with ℓ, is the main open step. Meanwhile the fitted relation ΔQ/Q = 0.056%/sc² (valid only for the tested range and parameters) and its uncertainty band give the sensitivity of the result to the choice.

TABLE VIII

Calibration summary

| Quantity | Value | Reference |
| --- | --- | --- |
| H89, H98 (baseline) | 1.01, 0.48 | [14], [15] |
| n̄/nG; βN | 0.63; 1.05–1.49 | [16], [17] |
| Beam torque estimate | 30–40 N m | 33 MW, 1 MeV [18] |
| Gain at that torque | 1.5% (sc = 0.5) | this work |
| ℓ / ρi (10 keV) | 16 | this work |

VII. DISCUSSION AND LIMITATIONS

A. Principal Findings

Three findings stand out. First, whether a shear-suppression closure is well behaved is not a numerical detail: for the local closure the converged answer is set by an edge condition that the physics does not supply, and a consistent discretization is unstable at fine resolution. A local-closure result should therefore be reported with its edge treatment, resolution, and eigenvalues; the smoothed closure removes the issue at the price of one new length, to which the gain is insensitive (Table V). Second, the admissibility conditions are a priori checks that cost nothing and are reproduced to ten digits. Third, in this model the diamagnetic shear changes the gain by well under one per cent at moderate threshold, and the sensitivity to the threshold, not the form of the closure, dominates the uncertainty.

B. Limitations

The limitations are those of a reduced model: no existence, uniqueness, or continuous-dependence theorem is proved for the nonlinear heat problem, so “ill posed” and “well behaved” rest on numerical evidence over the tested conditions; one-dimensional energy transport with fixed density, equal temperatures, and local alpha deposition; one-way coupling to rotation; no pedestal, edge-localized-mode, or magnetohydrodynamic physics; generic critical-gradient parameters (benchmarked only at the level of Section VI); an uncalibrated threshold and length; and no experimental data. The plasma current, elongation, and the Troyon limit enter only the calibration. A predictive claim would require gyrokinetic calibration of sc and ℓ, a self-consistent density and pedestal, and validation against experiment.

C. Implications for Compact, Cost-Constrained Tokamak Design

Compact devices have tight margins among confinement, field strength, auxiliary heating, and actuator capability, so a reduced transport model used to screen their operating regimes must not turn numerical artifacts into confinement gains. The present results bear on this only as a methodological prerequisite. The local closure is sensitive to the edge treatment and the resolution, whereas the regularized closure gave convergent solutions over the tested conditions; a predicted confinement improvement should therefore be accepted only after the flux admissibility, the grid convergence, and the sensitivity to the regularization length have been checked.

Auxiliary rotation drive is the clearest case. At the reactor-scale beam torque of Section VI-C the modelled gain is only 1.5% (sc = 0.5) to 9.3% (sc = 0.2), and larger gains need either stronger suppression or a torque far above that estimate. This says nothing about the torque a compact device requires, because size, field, beam energy, geometry, and momentum transport change the accessible regime.

A natural extension would apply the verified closure to a family of compact configurations that differ in major radius, minor radius, field, plasma current, and auxiliary power, and compare confinement time, fusion gain, actuator requirements, and operating margins under the same admissibility and convergence criteria; the one-dimensional model would first need geometric rescaling and equilibrium constraints. The relevant objective is not the largest modelled gain but an operating point that reaches adequate confinement with little auxiliary power and little sensitivity to the uncertain closure parameters. Weighing fusion output against auxiliary power, other power demands, and a normalized engineering cost would require a cost model, which is not part of this study.

The model is not sufficient to establish compact-device feasibility: it omits self-consistent equilibrium and stability, pedestal physics, current drive, magnet engineering, neutron shielding, and balance of plant, and the threshold and the length are uncalibrated against gyrokinetic calculations or experiment. The contribution is therefore a verified method for assessing closure sensitivity, not a prediction that a compact or low-cost tokamak can reach a given gain; neither plant-level net electric power nor cost is computed.

VIII. CONCLUSION

State-dependent shear-suppression closures need two things that are easy to overlook: an admissibility condition on the flux and a length that regularizes the shearing rate. With them the reduced model is verified, converges at second order, and gives a gain described over the tested range by the fitted relation ΔQ/Q = 0.056%/sc² with a documented uncertainty, while the local closure is numerically ill posed and its answers depend on the edge treatment. The admissibility conditions are exact and tested to ten digits. The baseline is L-mode-like (H89 = 1.01, a consistency check because the edge temperature is imposed), and neutral-beam torque of reactor size adds about 1.5%. The suppression threshold and the smoothing length are the quantities that a gyrokinetic calibration must supply before the gain can be used for design.

ACKNOWLEDGMENT

The author used Claude Sonnet 5.5 (Anthropic; model identifier claude-sonnet-5-5), accessed through the Claude Code environment of the Claude desktop application between 7 and 9 October 2026, to write and test the code, derive and check the closed-form results, run the numerical experiments, produce the figures, and draft the text. The author reviewed and edited all content, checked every reference against Crossref, and takes full responsibility for the paper. No funding was received.

CODE AND DATA AVAILABILITY

All code, tests, raw results (results.json), figure scripts, and the manuscript builders are at https://github.com/sandlerleon/tokamak-statedependent-closure (release v1.1.1) and archived at https://doi.org/10.5281/zenodo.23267847 (software, MIT licence); this manuscript is archived as a preprint at https://doi.org/10.5281/zenodo.23267849. No experimental data were used. A supplementary file contains the proofs, the numerical protocol, and extended tables.

REFERENCES

[1]	A. M. Dimits et al., "Comparisons and physics basis of tokamak transport models and turbulence simulations," Phys. Plasmas, vol. 7, no. 3, pp. 969–983, 2000, doi: 10.1063/1.873896.

[2]	M. Kotschenreuther, W. Dorland, M. A. Beer, and G. W. Hammett, "Quantitative predictions of tokamak energy confinement from first-principles simulations with kinetic effects," Phys. Plasmas, vol. 2, no. 6, pp. 2381–2389, 1995, doi: 10.1063/1.871261.

[3]	H. Biglari, P. H. Diamond, and P. W. Terry, "Influence of sheared poloidal rotation on edge turbulence," Phys. Fluids B, vol. 2, no. 1, pp. 1–4, 1990, doi: 10.1063/1.859529.

[4]	T. S. Hahm and K. H. Burrell, "Flow shear induced fluctuation suppression in finite aspect ratio shaped tokamak plasma," Phys. Plasmas, vol. 2, no. 5, pp. 1648–1651, 1995, doi: 10.1063/1.871313.

[5]	K. H. Burrell, "Effects of E×B velocity shear and magnetic shear on turbulence and transport in magnetic confinement devices," Phys. Plasmas, vol. 4, no. 5, pp. 1499–1518, 1997, doi: 10.1063/1.872367.

[6]	P. W. Terry, "Suppression of turbulence and transport by sheared flow," Rev. Mod. Phys., vol. 72, no. 1, pp. 109–165, 2000, doi: 10.1103/revmodphys.72.109.

[7]	R. E. Waltz, G. D. Kerbel, and J. Milovich, "Toroidal gyro-Landau fluid model turbulence simulations in a nonlinear ballooning mode representation with radial modes," Phys. Plasmas, vol. 1, no. 7, pp. 2229–2244, 1994, doi: 10.1063/1.870934.

[8]	S.-I. Itoh and K. Itoh, "Model of L to H-mode transition in tokamak," Phys. Rev. Lett., vol. 60, no. 22, pp. 2276–2279, 1988, doi: 10.1103/physrevlett.60.2276.

[9]	F. L. Hinton, "Thermal confinement bifurcation and the L- to H-mode transition in tokamaks," Phys. Fluids B, vol. 3, no. 3, pp. 696–704, 1991, doi: 10.1063/1.859866.

[10]	H.-S. Bosch and G. M. Hale, "Improved formulas for fusion cross-sections and thermal reactivities," Nucl. Fusion, vol. 32, no. 4, pp. 611–631, 1992, doi: 10.1088/0029-5515/32/4/i07.

[11]	J. P. Freidberg, Ideal MHD. Cambridge, U.K.: Cambridge Univ. Press, 2014.

[12]	J. Wesson, Tokamaks, 4th ed. Oxford, U.K.: Oxford Univ. Press, 2011.

[13]	P. Perona and J. Malik, "Scale-space and edge detection using anisotropic diffusion," IEEE Trans. Pattern Anal. Mach. Intell., vol. 12, no. 7, pp. 629–639, 1990, doi: 10.1109/34.56205.

[14]	P. N. Yushmanov et al., "Scalings for tokamak energy confinement," Nucl. Fusion, vol. 30, no. 10, pp. 1999–2006, 1990, doi: 10.1088/0029-5515/30/10/001.

[15]	ITER Physics Expert Group on Confinement and Transport et al., "Chapter 2: Plasma confinement and transport," Nucl. Fusion, vol. 39, no. 12, pp. 2175–2249, 1999, doi: 10.1088/0029-5515/39/12/302.

[16]	M. Greenwald, "Density limits in toroidal plasmas," Plasma Phys. Control. Fusion, vol. 44, no. 8, pp. R27–R53, 2002, doi: 10.1088/0741-3335/44/8/201.

[17]	F. Troyon, R. Gruber, H. Saurenmann, S. Semenzato, and S. Succi, "MHD-Limits to Plasma Confinement," Plasma Phys. Control. Fusion, vol. 26, no. 1A, pp. 209–215, 1984, doi: 10.1088/0741-3335/26/1a/319.

[18]	E. J. Paul, M. Landreman, F. M. Poli, D. A. Spong, H. M. Smith, and W. Dorland, "Rotation and neoclassical ripple transport in ITER," Nucl. Fusion, vol. 57, no. 11, Art. no. 116044, 2017, doi: 10.1088/1741-4326/aa7fa4.